Race ABS Diagnose

- To start the Diagnoses push “Start“

- To read failure push “Fehler lesen”, you will read a number code*

- To see failure discription as text push “Klartext”

- To erase failure push “Löschen”

* to indentify the number code read Failiure list or File Mk60BN2a.TXT in C:\Minidiag\PL2

Failure list (Stand: 13.08.02)

# Wheel 1	Sensor front left

$5D90	Electrical fault

$5D91	Continuity fault

$5D92	Periodic fault

$5D93	Speed comparison

$5D94	Phase length fault

$5D95	Signal fault

# Wheel 2	Sensor front right

$5DA0	Electrical fault

$5DA1	Continuity fault

$5DA2	Periodic fault

$5DA3	Speed comparison

$5DA4	Phase length fault

$5DA5	Signal fault

# Wheel 4 Sensor rear left

$5DB0	Electrical fault

$5DB1	Continuity fault

$5DB2	Periodic fault

$5DB3	Speed comparison

$5DB4	Phase length fault

$5DB5	Signal fault

# Wheel 3	Sensor rear right

$5DC0	Electrical fault

$5DC1	Continuity fault

$5DC2	Periodic fault

$5DC3	Speed comparison

$5DC4	Phase length fault

$5DC5	Signal fault

$5DE0	BBV FA not initialized

$5DE1	BBV RA not initialized

$5DE2	critical Brake Block Thickness FA is reached

$5DE3	critical Brake Block Thickness RA is reached

$5DF0	Pump motor defect

$5DF2	Hardware Fault (ECU/ EMV)

$5DF4	Voltage low

$5DF5	Internal Fault (ECU/ Valve-Coils/ Sensor Supply)

$5DF7	Voltage high

$5E11	CAN Failure (Bus Off or wrong CAN-Revision)

$5E14	CAN Engine Timeout (No message for more than 500ms)

$5E15	CAN Gear Timeout (No message for more than 500ms)

$5E16	CAN Instrument Cluster Timeout (No remote frame received for 5 times or no message for more than 1s)

$5E18	CAN Motronic Plausibility (No Q_ASC toggle more than 500ms)

$5E19	CAN Engine Torque not adjustable (STAT_MD_E=3)

$5E1A	CAN Engine Signal Error (F_N_MOT=1 for more than 500ms or W_FPDK=$FF for more than 500ms)

$5E1E	CAN SWA Timeout (No message for more than 100ms or LWS_COUNT= const for more than 100ms)

$5E1F	ECU not initialized (No or wrong chassis number)

$5E20	Pressure_Transducer_1 el. defective or noisy

$5E21	Pressure_Transducer_2 el. defective or noisy

$5E24	Pressure_Transducer_1/2  Plausibility

$5E26	Sensor Supply Voltage Fault

$5E2F	Temperature Sensor Fault

$5E30	Lateral Accelerometer Sensor el. defective

$5E32	Lateral Accelerometer Sensor Signal Fault

$5E38	Yaw Rate Sensor el. defective

$5E3C	Yaw Rate Sensor Signal Fault

$5E40	SWA Sensor CAN invalid or Signal Fault

$5E43	SWA Sensor Internal Fault

$5E4E	ESP Sensor offset check failed (P-Sensor, YR-Sensor, AY-Sensor, SWA-Sensor)

$5E4F	ESP Continuous Operation

$5E58	Unexpected LWS-Message (TC-System in a car with DSC equipment)

$5E59	Parameter Error (Wrong Gearbox type)

$5E5B	TC Switch defective

$5E5D	BFL as too low detected

$5E5E	BLS defective

Failure message (Stand: 25.03.02)

Aufbau des Receive-Telegramms

Byte

1 = xx IDByte 	11 111111	ID Telegrammlänge

2 = F1 Tester ID

3 = 29 ECU ID

4 = 2+3*n Längen-Byte (meint Anzahl Fehler*3 + 2Byte)

5 = 58 ServiceNummer (SN)

6 = xx Fehlerzähler

7 = xx Byte   1. Fehler

8 = xx Sender 1. Fehler

9   folgende Bytes (bis max 5 Fehler)

n= CS

Erläuterung :

Die TEVES ABS/ASC-SG´s mit BN2000 können grundsätzlich alle aufgetretenen Fehler abspeichern  beim Auslesen werden allerdings nur die aktuellsten 5 angezeigt. Bei einer Fehleranzahl größer 5 wird jeweils der Fehler mit dem ältesten Stand überschrieben.

Race ABS Sensor Calibration

- To calibrate the ESP sensors (longitudinal and lateral acceleration, MC pressure, steering) push “DSC Abgleich”.

- choose register MK60

- choose file Adj_DSC6.sed

- The calibration is runs automatic

Race ABS Brake Bleeding

Before starting ABS bleeding do a conventional bleeding on the car. When you have a more or less acceptable pedal feel start the ABS bleeding procedure.

- To support the brake bleeding when ABS is installed press “Aktionen”! Choose “Ansteuerrung Sonderventile mit Pumpe” (pump running building pressure).

- Than actuate brake pedal and press “Ansteuerung Ausslassventil (the pedal will drop to the floor when the valves are actuated. Keep the foot on the brakepedal.

- Than actuate “Einlassventile mit Pumpe”! The pump runs and builds pressure against the driver foot. The pedal raise up.

Repeat this 2 or 3 times. Then do another conventional bleeding

If the pedal still is not ok repeat the ABS bleeding.